Flood estimation – calculation record

Site/project name:

Date:

Introduction

This document is a supporting document to the Natural Resources Wales Flood Estimation Technical Guidance Note V4, published in 2026. It provides a template for recording calculations and decisions made during flood estimation. It will often be complemented by more general hydrological information given in a project report. The information given here should be enough to enable the work to be reproduced.

Note 1: Table, content or page layout can be adapted to best present relevant information. Additional rows should be added to, or removed from tables as appropriate.

Note 2: Probability of flood occurrence is traditionally expressed within Hydrology as a Return Period, this is the average time between years with at least one larger flood. It can also be expressed as Annual Exceedance Probability (AEP), and this is often more appropriate to use when communicating with non-hydrologists. Return Period has been retained within this document but can be replaced with AEP if wished.

Contents

1.	Method statement	3

2.	Locations where flood estimates are required	8

3.	Statistical method	12

4.	Revitalised flood hydrograph (ReFH) method for peak flow estimation	17

5.	Revitalised flood hydrograph (ReFH) method for model inflow hydrographs	18

6.	Final peak flow and hydrograph estimates	21

Appendix: supporting information	26

Approval

|  | Name | Qualifications | Date |
| --- | --- | --- | --- |
| Prepared by: |  |  |  |
| Checked by: |  |  |  |
| Approved by: |  |  |  |

Abbreviations

| AEP | Annual exceedance probability |
| --- | --- |
| AMAX | Annual maximum flow |
| AREA | Catchment area (km2) |
| BFI | Base flow index |
| BFIHOST19 | Base flow index derived using the revised (2019) HOST soil classification |
| BFIHOST19_Scaled | Base flow index derived using the revised (2019) HOST soil classification, updated in 2025 to better account for correlations with FARL2015 |
| DPLBAR | Mean drainage path length (km) |
| DPSBAR | Mean drainage path slope (m/km) |
| FARL | FEH index of flood attenuation due to reservoirs and lakes |
| FARL2015 | FEH index of flood attenuation due to reservoirs and lakes using 2015 spatial data |
| FEH | Flood Estimation Handbook |
| FPEXT | Floodplain extent |
| HOST | Hydrology of soil types soil classification |
| NRFA | National river flow archive |
| POT | Peaks over a threshold |
| QMED | Median annual maximum flow (with Annual Exceedance Probability of 50% / return period 2 years) |
| ReFH | Revitalised flood hydrograph – used for rainfall runoff method |
| SAAR6190 | Standard average annual rainfall (mm) for years 1961 to 1990 |
| SAAR9120 | Standard average annual rainfall (mm) for years 1991 to 2020 |
| SPR | Standard percentage run-off |
| Tp | Time to peak |
| URBEXT2000 URBEXT2015 | Index of urban extent using 2000 spatial data Index of urban extent using 2015 spatial data |
| WINFAP | Windows Frequency Analysis Package – can be used for FEH statistical peak flow method |

# Method statement

## 1.1	Overview of requirements for flood estimates

| Item | Comments |
| --- | --- |
| Give an overview which includes: purpose of study names of river/s location number of calculation points and if peak flows or hydrographs are required previous relevant calculations availability of flood history |  |

## 1.2	Overview of catchment

| Item | Comments |
| --- | --- |
| Brief description of catchment, including key features needing consideration or reference to section in accompanying report. Map/s should be presented here or in section 2.1 of this report. |  |

## 1.3	Source of flood peak data

| Item | Comments |
| --- | --- |
| Was the NRFA Peak Flows dataset used? If so, which version? | Version 14, released August 2025. This contains data up to water year 2023-24 at most sites. (Update text if needed) |

## 1.4	Gauging stations (flow or level)

Within, or near to, the study area. Most stations will be included on National River Flow Archive (NRFA), but other stations may also be available.

| Watercourse | Station name | NRFA number | Grid reference | Catchment area (km2) | Location relative to study area (eg, within), note any significant differences in catchments (eg URBEXT) |
| --- | --- | --- | --- | --- | --- |

## 1.5	Data available at each gauging station

| Station name or number | Start and end date on NRFA | Suitability (Pooling/ QMED/ Neither)? | Update for this study? | Comments on data availability (quality is covered in next section). If not a QMED or pooling station then describe how data will be used e.g. for Tp calculation or QMED calculation from daily mean flow |
| --- | --- | --- | --- | --- |

## 1.6	Data quality at each gauging station

| Station name or number | Comments on rating quality focusing on QMED and above e.g. degree of extrapolation, availability of recent flow gaugings, amount of scatter in the gaugings, bank full level and out of bank flow |
| --- | --- |

## 1.7	Other data available and how it has been obtained

Consider flood history and local data if available.

| Type of data | Source of data | Details and reference/link to relevant reports |
| --- | --- | --- |
| Historic flood data |  |  |
| 15-min river level/flow and rainfall data for events (if carrying out Tp or ReFH analysis) |  |  |
| Results from previous studies |  |  |

## 1.8	Initial choice of approach

| Item | Comment |
| --- | --- |
| Outline the conceptual model. Address questions such as: Where are the main sites of interest? What is likely to cause flooding at those locations? (e.g. peak flows, flood volumes, combination of peaks, groundwater, snowmelt, tides) Might those locations flood from runoff generated on part of the catchment only e.g. downstream of a reservoir? |  |
| Any unusual catchment features to account for? For example: highly permeable (BFIHOST19_Scaled> 0.65) urbanised – consider choice of method carefully; consider artificial drainage and storm sewer inflow and outflows major reservoir influence – consider flood routing extensive floodplain storage – consider choice of method carefully |  |
| Initial choice of method(s) and reasons Will the method often known as FEH Hybrid be used for peak flow estimates? (Statistical for QMED to 1% AEP, then growth factor from ReFH applied to 1% event for events to 0.1% AEP) If not appropriate, describe why and give details of the other method/s to be used. What method will be used for hydrograph calculation? Will the catchment be split into sub-catchments/intervening areas? If so, how will flows for intervening areas be estimated? |  |
| Software to be used including version number/s: |  |

# Locations where flood estimates are required

## 2.1	Map of study area, including subject site(s) and gauging stations (where applicable).

## 2.2	Summary of subject sites

The table below lists the locations of subject sites. Use site codes in all subsequent tables to save space.

| Site code | Watercourse | Site Name (description) | Easting | Northing | AREA (km2) | Peak flow, hydrograph or both required? |
| --- | --- | --- | --- | --- | --- | --- |

## 2.3	Checking catchment descriptors

| Item | Comment |
| --- | --- |
| Record how catchment boundary was checked Describe any changes Add maps if needed |  |
| Method for updating URBEXT2000 (used in Rainfall Runoff method) and/or URBEXT 2015 (used in Statistical Method): OS 1:50,000 Map UKCEH Landcover Map OS Open Built Up Areas Dataset If the catchment is rural, it is likely be appropriate to keep URBEXT values as they are or to use the General Urban expansion factor calculated for year of interest |  |
| Record how other catchment descriptors were checked, especially soils Describe any changes Include a before and after table if necessary |  |

## 2.4	Important catchment descriptors at each subject site (incorporating any changes made)

| Site code | AREA (km2) | BFIHOST 19 | BFIHOST 19_Scaled | FARL | FARL 2015 | SAAR6190 (mm) | SAAR9120 (mm) | URBEXT 2000 | URBEXT 2015 | Changes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Statistical method

## 3.1	Donor stations and QMED adjustment factors

Note that donor catchments will usually be rural but may be urban provided this is accounted for in the adjustment process. Include a map and any notes on QMED calculation as appropriate.

| Station | Record Length | Is station hydrologically connected to subject site/s? Ie, upstream / downstream | QMED Observed (A) (m3/s) | QMEDUrban Catchment descriptors (B) (m3/s) | Adjustment ratio (A/B) | Chosen or rejected, with reasons and other comments |
| --- | --- | --- | --- | --- | --- | --- |

## 3.2	Overview of estimation of QMED at each subject site

There is a presumption that catchment descriptor with donor transfer method will be used. If an alternative method was utilised, add a table to record details and reasons as appropriate.

Urban adjustment procedures should be applied regardless of whether the subject site is rural or urban.

| Site code | Watercourse | QMEDurban catchment descriptors (m3/s) | NRFA numbers for donor stations used | Distance moderation applied? | QMED adjustment ratio | QMEDurban donor adjusted (m3/s) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |

## 3.3	Derivation of pooling groups

Several subject sites may use the same pooling group.

The composition of pooling groups should be presented in the Appendix.

| Pooling group name | Site for which pooling group was derived | If applied to more than one site, list their codes | Method: Single Site / with History / Pooled? Include reasons for choice of method | Changes made to default pooling group, with reasons Include any sites that were investigated but retained in the group |
| --- | --- | --- | --- | --- |
| Were non-flood year adjustments made to any stations? if so give details |  |  |  |  |
| URBEXT2015 threshold used to create pooling group(s). |  |  | We recommend that this is set to 0.3 to make maximum use of available data and that growth curves are deurbanised appropriately |  |
| Have pooling group growth curves been deurbanised? |  |  |  |  |

## 3.4	Derivation of flood growth curves at subject sites

A pooling group derived at one location can be applied to estimate growth curves at several ungauged sites. However, each site may have a different urban adjustment, and therefore different growth curve parameters.

Any relevant frequency plots, particularly showing any comparisons between single-site and pooled growth curves (including flood peak data on the plot) should be shown in an Appendix.

| Site code | Pooling group name | Distribution used and reason for choice | Was an urban adjustment made? | Growth factor for 100-year return period (1% AEP) event |
| --- | --- | --- | --- | --- |

## 3.5	Flood estimates from the statistical method

| Site code | Flood peak (m3/s) for the following return period or AEP events |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 50% | 5 20% | 10 10% | 20 5% | 30 3.3% | 50 2% | 75 1.3% | 100 1% | 200 0.5% | 500 0.2% | 1000 0.1% |

# Revitalised flood hydrograph (ReFH) method for peak flow estimation

This section records calculations for peak flow estimates and will generally use default parameters.  If different calculations are subsequently made for hydrographs, details should be recorded in section 5.

## 4.1	Design events for ReFH method for peak flow estimation

| Site code | Changes made to drained area descriptors? | Season of design event (summer or winter) | Recommended Storm duration (hours) | Comments |
| --- | --- | --- | --- | --- |

## 4.3	Peak flow estimates from the ReFH method

Urban estimates should be used for all sites

|  | Flood peak (m3/s) for the following return periods (in years) or AEP |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Site code | 2 50% | 5 20% | 10 10% | 20 5% | 30 3.3% | 50 2% | 75 1.3% | 100 1% | 200 0.5% | 500 0.2% | 1000 0.1% | 100:1000 1%:0.1% ratio |
| Describe how do peak flows compare to statistical estimates (figures presented in section 6.1) |  |  |  |  |  |  |  |  |  |  |  |  |

# Revitalised flood hydrograph (ReFH) method for model inflow hydrographs

## 5.1	Parameters for ReFH model for model inflow hydrographs

This section records calculations for model inflow hydrographs, parameters may have been calibrated and storm durations changed.

If parameters are all estimated from catchment descriptors, they are easily reproducible, so it is not essential to record them here –Just enter ‘all’ under site code and ‘Catchment descriptors’ under method. Table can be amended as needed.

| Site code | Details of method Catchment descriptors (CD) Tp (Time to peak) calculation Optimisation (Calibration Utility) | CD Tp (hours) | Adjusted Tp if different (hours) |
| --- | --- | --- | --- |
| Description of any Tp calculation or calibration work, add references to other documents where appropriate |  |  |  |

## 5.2	Design events for ReFH method for model inflow hydrographs

| Storm duration (hours) | ARF | Source of Storm Duration and ARF | Why Chosen |
| --- | --- | --- | --- |
| Were hydrographs scaled to alternative peak flow estimates? If so, give details |  |  |  |
| Provide link/reference to location of hydrographs or provide in appendix |  |  |  |

# Final peak flow and hydrograph estimates

## 6.1	Comparison of peak flow estimates from different methods

This table compares peak flows from the ReFH method, FEH Statistical method and any available previous study at each site for two key return periods. Note and explain any significant difference from previous studies.

|  | QMED (50% AEP) |  |  |  | 100-year return period / 1% AEP |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Site code | Statistical 2025 | ReFH | Previous Study | Comment | Statistical 2025 | ReFH | Previous Study | Comment |

## 6.2	Final peak flow estimates

|  | Flood peak (m3/s) for the following return periods (in years) or AEP |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Site code | 2 50% | 5 20% | 10 10% | 20 5% | 30 3.3% | 50 2% | 75 1.3% | 100 1% | 200 0.5% | 500 0.2% | 1000 0.1% |  |
| State choice of method, i.e. - Hybrid approach (Statistical for QMED to 100 year, ReFH growth factor from 100yr applied to statistical 100yr, for rarer events) - Statistical - ReFH |  |  |  |  |  |  |  |  |  |  |  |  |

## 6.3	Uncertainty

Give what information you can on uncertainty in the flood estimates

| Site Code | 95% confidence interval factor for QMED (eg 0.48-2.1 for ungauged locations with 0 donors) | 95% confidence interval factor for 100 year (1% AEP) flood | Comments, include comments on QMED estimation method, gauging station relative location/s, record length, data quality, consistency between stations, flood history and catchment characteristics |
| --- | --- | --- | --- |

## 6.4	Hydrographs for modelling

| How were these calculated, for example, scaling ReFH hydrographs to final flow estimates, adjusting Cini? include link/reference to hydrographs. |  |
| --- | --- |
| How will flows be applied in the model. If intervening areas are used, will hydrographs be adjusted to better match downstream flows, or will best estimate inflows be used and resulting downstream flows accepted? |  |

## 6.5	Checks

| Are the results consistent, for example at confluences? |  |
| --- | --- |
| What do the results imply regarding the return periods of floods during the period of record? |  |
| What is the 100-year growth factor? Is this realistic? (The guidance suggests a typical range of 2.1 - 4.0) |  |
| If 1000-year flows have been derived, what is the range of ratios for the 1000-year flow over 100-year flow? |  |
| What is the range of specific runoffs (l/s/ha)? Are there any inconsistencies? |  |
| How did the results compare with those of other studies? Explain any differences and conclude which results should be preferred |  |
| Are the results compatible with the longer-term flood history? |  |
| Describe any other checks on the results |  |

## 6.6	Assumptions and limitations

| List the main assumptions made specific to the study |  |
| --- | --- |
| Discuss any particular limitations For example, applying methods outside the range of catchment types for which they were developed |  |
| Comment on the suitability of the results for future studies For example, at nearby locations or for different purposes |  |
| Give any other comments on the study For example, suggestions for additional work |  |

# Appendix: supporting information

Please include details of your pooling group(s)

## Pooling group composition

## Additional supporting information